NATPLOT.DOC  (README)  Version (23)  --  24 APR 05

TABLE OF CONTENTS

-----------------

0.  BUSINESS STUFF

0.1  PURPOSE

0.2  COPYRIGHTS

0.3  FEEDBACK

0.4  URLs

1.  GETTING STARTED

1.1  QUICK START

1.2  INSTALLATION

1.3  FILE USAGE

1.4  CONTROL FILE (NATPLOT.CTL)

2.  MENU INFO

2.1  NAT TRACK FILES

2.11   PACOTS FILES

2.2  PROCEDURE FILES

2.3  PRINTER

2.4  DISPLAY

2.5  MAP

2.55   TEST PATTERN

2.6  PROJECTIONS

2.7  DATA

2.72   USER.TXT

2.75   TERMINATOR

2.76   TIME ZONE

2.77   Y2K

2.8  DAFIF

2.8.1  RUNWAY DISPLAY

2.8.2  AIRWAY DATA

2.9  PROPERTIES

3.  NON-MENU INFO

3.1  TOOL BAR

3.2  MOUSE TRACKING

3.3  HOT KEYS/CLICKS

3.4  MAGNETIC VARIATION

3.5  PROGRAMMING

4.  PROBLEMS

4.1  SID CIRCLES

4.2  EXTRA AND MISSING LINES

4.3  NAT/PACOTS TRACKS NOT DRAWN

4.4  WAYPOINT DATA OVERPRINTS

4.5  COURSE INTERCEPTS THAT DON'T

4.6  ERROR MESSAGES

4.7  FAILS ON START UP

4.8  SLOW DISPLAY UPDATES

5.  COMING ATTRACTIONS

6.  VERSION HISTORY

-------------------------------------------------------------------

|                    CHAPTER 0 - BUSINESS STUFF                   |

-------------------------------------------------------------------

0.1 PURPOSE

-----------

This version draws maps showing:

1) NAT Track messages.

2) PACOTS Track Notams available on the DoD web site.

3) DAFIF(tm) world-wide navigation database.  Display only

using custom files, no look-ups yet.

4) Fixed oceanic tracks for several areas, plus a few routes

through Russia and China.  These routes have not been

updated, but will be retained until I make the DAFIF

airways show the same information.

5) PS1 1.3 procedure files, PS1 1.2 waypoint/navaid files.

0.2 COPYRIGHTS

--------------

NatPlot is no longer copyrighted.  Concerns from the early

days of the Internet are no longer valid.

As you might expect - this software is provided on an "as is" basis,

and comes with absolutely no warranties whatsoever.  Furthermore,

your mileage may vary, offer void in some locations, send before

midnight tomorrow, batteries not included ...

The intended audience is ATC and Flight simmers.  Other use is

at your own risk.  Navigation data may not be current or accurate.

DAFIF(tm) is a product of the National Imagery and Mapping Agency.

See Help | DAFIF for details.

PS1 is copyright by Hardy Heinlin.

PKZIP/PKUNZIP is copyright by PKWARE.  See their web site for

shareware usage.

The USGS magnetic variation programs are public domain.

0.3 FEEDBACK

------------

I am interested in your comments and suggestions.  So far, the

number of e-mails has been lower than I expected, it has been

easy to respond, and the suggestions have been good ones.

(See if your idea is already listed below in chapter 5.)

When I am out of town, I usually do not pick up e-mail - might be

4 or 5 days.

Bob Raemer

0.4 URLs

--------

As of September, 2003:

NatPlot and associated data files are posted at:

http://ourworld.cs.com/bobraemer/ownnav/index.htm

Nat Track messages are posted at the Glideslope site:

http:/www.natroutes.glideslope.de/html/nats.php3

Nat Track and PACOTS messages are now available from the DoD Notam

site, https://www.notams.jcs.mil

According to the Policy page, this is public information and may

be distributed.  The following link seems to go to the same site:

https://www.notams.faa.gov.

If you type the address into your browser, you probably need to

include the "https://" part, or you may only get "http://".

The Digital Aeronautical Flight Information File is a

world-wide database, 20 Mb zipped, revised every 28 days.

It can be downloaded from:

https://164.214.2.62/products/digitalaero/index.cfm

Russian/Polar route information was available from several sites.

The www.racgat.com site seems to be deactivated.  Try a search

on RACGAT to get individual documents.

Magnetic Variation programs (in Fortran) were obtained

from:  http://geomag.usgs.gov

A distance/bearing program called inverse.for, which uses an

ellipsoid model for the Earth's surface, is available at:

http://www.ngs.noaa.gov/PC_PROD/Inv_Fwd/source/source.zip.

PKUNZIP is available at:  http://www.pkware.com

My e-mail address is:  bobrae@attglobal.net

-------------------------------------------------------------------

|                    CHAPTER 1 - GETTING STARTED                  |

-------------------------------------------------------------------

1.1 QUICK START

---------------

***************************************

******  REQUIRES:                ******

******      WINDOWS 95/98/ME/XP  ******

******      PKUNZIP              ******

***************************************

My development system is ME, and I will do limited testing on

95/98/XP.  Users have reported that this program will not run on

NT/2000.

Assuming you have a default installation of PS1 on drive C:, or

do not have PS1:

(1)  Create a folder on drive C: called NATPLOT

(2)  copy NATPLOT.ZIP to that folder

(3)  Unzip it.

(4)  Download the latest Nat Track Message (see Section 2.1)

and/or unzip the sample message files.

(5)  Download and unzip the DAFIF excerpts (DAFx, WPTx, RWY0).

(6)  run NATPLOT.EXE

If that doesn't work, or you need more detailed information,

or to use other folders, see section 1.2.

1.2 INSTALLATION

----------------

This program is not at the point where it has fully automatic

installation.  It may never be.  You must know enough about

Windows 95/98/ME or MS-Dos to create a sub-directory (folder), copy

files, and set up shortcuts if you wish.  You also need a copy

of PKUNZIP or another un-zip program.

NATPLOT.ZIP contains 5 files:

NATPLOT.DOC    You're reading it

NATPLOT.EXE    The program

NATPLOT.CT0    Information about control file NATPLOT.CTL

NATPLOT.DAT    Internal fixes, route data, coastlines

NATPLOT.FIX    Ascii list of internal fixes

USER.TXT       Sample user data file

You can unzip these sample track messages into your data folder(s),

or just download current ones from other sources.

NATSAMP.ZIP    Sample Nat Track files.

PACSAMP.ZIP    Sample Pacots Track files.

The DAFIF data is in several zip files:  DAFx, WPTx, RWY0, RADIALx.

The remaining BIN (binary format) files will be replaced

with text files.

DAFN.BIN       Navaids

DAFA.BIN       Airports

RWY0.TXT       Runways

WPTA.TXT       Waypoints - Asia

WPTE.TXT       Waypoints - Europe

WPTN.TXT       Waypoints - North America

WPTS.TXT       Waypoints - Southern Hemisphere

RADIALK.TXT    Airway details - North America, Pacific

RADIALX.TXT    Airway details - rest of world

If your file locations are not the same as the default ones,

or if you want to change some of the start-up values, you will

need to create a control file called NATPLOT.CTL.  Do this with

any text editor (e.g. Notepad).  Data line formats can be found

in NATPLOT.CT0.  Distribution zip files will contain the 'CT0'

file, and will not over-write your 'CTL' file.

You can put these files anywhere you wish -- but keep the EXE,

CTL, and DAT files in the same folder.

The "right" way to use this information is via the Windows Registry.

That's on my list of things to do - just ahead of learning Cobol.

Here are the steps for a default installation:

(1) Create a new folder on drive C: called NATPLOT

(2) Copy NATPLOT.ZIP to that folder.

(3) Unzip it:  pkunzip natplot

(4) (Optional) Create a shortcut to NATPLOT.EXE and put it where

you put your other shortcuts.  For info on how to do that,

click the Start button, select Help, and see topic

"Shortcuts".  If you use the Wizard to create the shortcut,

the 'Start in' entry should contain C:\NATPLOT\ (the path

part of the 'Target' entry).  If not, you will have to set

it yourself.

(5) (Optional) Create an entry in Start|Programs.  For info on

how to do that, click the Start button, select Help, and

see topic "Start menu, adding programs to".

(6) (Optional) In the C:\NATPLOT\ folder, unzip the sample track

message files. The files in PACHTM.ZIP will be copied to

the folder C:\NATPLOT\PACOTS if you use the -d option

(restore directory structure):  pkunzip -d pachtm

In the windows version of PKZIP, under Extract, Preferences,

Options, check "Create Directory".

(7) Download the latest Nat Track file from the Glideslope or

DoD site, and save it in the C:\NATPLOT\ folder.  Details in

section 2.1.

Download the latest Pacots notam from the DoD web site,

and save it in the C:\NATPLOT\PACOTS\ folder.  Details in

section 2.11.

(8) Download & unzip the NatPlot excerpts of the DAFIF data files

to the NATPLOT folder. Other locations are possible by using

a control file entry.

(9) If your NatPlot or PS1 files are in different locations, or

you want to change the start-up values, create NATPLOT.CTL

with appropriate entries.  Info in NATPLOT.CT0.

1.3 FILE USAGE

--------------

NatPlot displays the oceanic track files you have downloaded, and

what is in your PS1 procedure files.  Since you may have modified

the distribution files, or downloaded different user-contributed

files, I may not be able to duplicate your problems.  Please

do NOT e-mail your data files to me.

NatPlot opens all files in read-only mode, and closes them as

soon as the display is updated.  The only time it may interfere

with the operation of other programs (or vice-versa) is during

the brief time the data files are being read.

The only disk output from NatPlot goes to a file called DUMP.TXT

that I use for debugging.  Output is enabled by an undocumented

control file entry, so you should not see this file (unless there

is a bug somewhere).  NatPlot will not change anything else on

your hard drive.

1.4 CONTROL FILE (NATPLOT.CTL)

------------------------------

The control file is an optional text-format file that allows

you to change the default behavior of NatPlot.  More details

can be found in the sample file NATPLOT.CT0 that is included

in NATPLOT.ZIP, and in other sections of this document.

You will need to use the control file if your files are not

in the default locations.  Other than that, it is optional.

The other items it controls can be changed with menu items

each time you run the program.

-------------------------------------------------------------------

|                     CHAPTER 2 - MENU INFO                       |

-------------------------------------------------------------------

2.1 NAT TRACK FILES

-------------------

Go to the Glideslope web site's Nat Tracks page, or the DoD site,

and use your browser's File|Save As function to save the Daily

Nat Track Message.  Save it in folder C:\NATPLOT\, or elsewhere

if you have made a 'NatTrackDir' control file entry.

When saving the file, I use the date of the message for the file

name in YYMMDD format.  If you followed the suggestions above,

the 22 Jan 99 file would be named 990122.txt (or 990122.htm), and

saved in folder C:\NATPLOT\.  If you save everything forever, you

may want to call it 19990122.txt.

If you are using Internet Explorer you may save it in either HTML

or plain text (TXT) format.  Text format is smaller and will be

easier to edit, if either of those things are important to you.

If your browser saves several files, including a folder with GIF

files - in the "Save as Type" box use "Web Page, HTML only",

instead of "Web Page, complete", or try:

-- Edit|Select All in your browser.

-- run Notepad, and use Edit|Paste.

-- save the Notepad file.

NatPlot is not a real HTML-decoder.  A better term might be

HTML-ignorer.  However, a tag (e.g. <BR>) is expected between

logical lines.  NatPlot will get confused if there are no tags

in what it thinks is an HTML file.

If you are using Netscape, save it in HTML format.  Users have

reported problems using NetScape's Save As with text format

(1999 version - you may want to try it again).

I saved HTML format from Mozilla Firefox 1.0 by selecting some

text in the frame containing the routes, right-clicking, selecting

"This Frame ..." and "Save Frame as ...".   This seemed to work,

but I only tried it once.

I tried to save in TXT format from Mozilla Firefox 1.0 with the

same procedure, except selecting text format in the file name dialog

box.  This added an HTML extension on the file, but actually seems to

save it in text format.

Other browser shells provided by various Internet Service

Providers may require other procedures.

I normally save in TXT format using Notepad as follows:

-- run Notepad

-- display the routes from the DoD site in a browser

-- select (using a menu item, or drag over it)

-- copy

-- switch to Notepad

-- paste

-- save the file

You may use the control file entry NatFileType to set which type

of file is initially displayed in NatPlot's file-open dialog box.

The default in version 0.9 and later is HTML.  If you are using

IE, you may want to change the control file entry to TXT.

Selecting FILE | READ NAT TRACKS... should get you a dialog box

showing the files in C:\NATPLOT\ that match the NatFileType.

If not, you probably have an incorrect 'NatTrackDir' entry in

NATPLOT.CTL.  You may use the drop down list in the 'Files of type'

box to see other file types.

NatPlot looks for lines whose first non-blank character is a

single letter followed by recognizable fixes, and ignores almost

everything else.  From time to time the track data lines begin with

"NAT " or "NAT-" followed by the single letter track designator.

This has happened often enough that NatPlot now looks for this

as well.  (The messages are obviously not designed for computer

consumption.)

It looks for 'TRACKS FLS' and prints the associated information

at the left edge of the display.  It ignores any text between

'<' and '>', assuming this is an HTML instruction.  It ignores

lines over 1500 characters, in case it gets lost in the middle

of some unexpected HTML formatting.

A single letter in the first half of the alphabet designates a

westbound route, second half for eastbound.  Westbound routes are

drawn in black, eastbound routes in red.

It is possible for portions of the eastbound NAT Tracks to coincide

with the westbound tracks on a given day.  The tracks are offset to

the north or south by one pixel on the video display, and two pixels

on the printer, so both can be seen.

The fixed tracks (e.g. Central East Pacific) follow the same plan,

with both red and black lines drawn for two-way tracks.

I have seen several errors in posted Nat Track files, producing

zig-zag routes, among other things.  Navaid identifiers are usually

used, except the facility name TIREE is often used instead of TIR.

(NatPlot catches this.)  Sometimes fix names are obviously

misspelled.  Sometimes garbage (non-printing) characters get into

the posted files, causing an apparently valid fix to be treated

as unknown.

NatPlot knows most of the named fixes currently in use.  However,

these may change.  If an unknown fix is encountered, the processing

of that route stops (no error message).

"Recognizable fixes" include NatPlot's internal Waypoints, Stations

and Airports, 2 or 3 digit lat/longs assuming north/west (50/40),

seven character lat/longs (40N170E), five character FMS lat/longs

(5040N), and full lat/longs (S1234.56/E12345.67).  Recognizable

fixes for Nat Track display do NOT include items in the PS1 data

files.

You could create your own file(s) of routes for locations other

than the North Atlantic, although with a limited number of named

fixes this would be cumbersome to use.  Remember to use a single

letter to begin each line that you want drawn.

2.11 PACOTS FILES

-----------------

PACOTS (Pacific Organized Track System) routes are available

in real time from the Dept. of Defense web site listed in the URL

section above.  That page now has a button to access PACOTS

messages, so you no longer need to use the facility identifiers.

One way to save the messages in text format:

(1) open a Notepad session.

(2) right-click within the message, and select "Select All"

(3) right-click within the message, and select "Copy"

(4) right-click within Notepad, and selete "Paste"

(5) Save the Notepad file.  Don't add ".txt" to the file name.

Your browser may also permit saving the files with a "File|Save As"

function.  If your browser saves several files, including a folder

with GIF files - in the "Save as Type" box use "Web Page, HTML only",

instead of "Web Page, complete".

Sometimes the "Save As" will only save the title section of the

page, but no data lines.  Another possibility is right-clicking

the data area and selecting "View Source".  This may put the

HTML code for that section of the page into Notepad, which you

can then save as a file.

I save PACOTS messages in a separate folder from the NAT messages

(C:\NATPLOT\PACOTS), but they could be kept in the same folder.

The various routes are updated at different times during the day,

so you may download different messages for the same date.  I have

been using the date in YYMMDD format, followed by the letter 'P',

and a number indicating which message this is of the current day.

The second message from 25 Dec 02 would be named 021225P2.TXT.

I am not sure of all the formatting details of these track

messages, but this is enough to get started.

Oakland Center track messages contain "(TDM TRK ", the track

designator, some date/time groups, possible "AMDT" or

"REPLACE xxxx" groups, the fixes that make up the route, "RTS/xxxx"

where xxxx is the departure airport, and what appears to be

transition routes and remarks.  NatPlot uses the track designator

and the fixes, and ignores the rest.

Tokyo Center track messages contain "TRACK x. FLEX ROUTE : ", the

fixes that make up the route, followed by "JAPAN ROUTE" or

"RCTP/VHHH ROUTE", and other information.  Sometimes ROUTE is RTE.

Although the main subject is about something else, the following

document contains a description of the track designators:

http://www.faa.gov/ats/ato/rnp.htm.  In addition, tracks 14 and 15

are now used from RCTP/VHHH to KSFO/KLAX.

The "posted by" times are what I observed in 1999.  In late 2002,

you can get one message containing most of the tracks for a given

date at around 0100 UTC.

Posted

Track Designator  Origin/Destination             Effective    By

----------------  -----------------------------  ---------  ------

1-4             Tokyo to US West Coast          09Z-16Z     00Z

8               Japan to Dallas                 07Z-23Z     00Z

11, 12          Tokyo to Honolulu               12Z-16Z     00Z

A-B             Honolulu to Japan               19Z-08Z     15Z

C-G             US West Coast to Japan          19Z-08Z     15Z

H-I             KSFO/KLAX to Taipei/Hong Kong   19Z-08Z     15Z

J-K             KSFO/KLAX to Taipei/Hong Kong   05Z-21Z     00Z

L               Los Angeles to Manila           05Z-21Z     00Z

M               Dallas to Tokyo                 19Z-08Z     15Z

I've added a number of fixes to the database to accommodate these

routes.  It may not be practical to include all of the possible

inland fixes to/from U.S. cities.  Eventually the fix look-ups

will be in the DAFIF database, although the custom database will

still be needed to display old files using obsolete fixes.

About one message out of ten from Tokyo in 1999 included airway

designators.  NatPlot ignores NOPAC route designators, and draws a

straight line between the named fixes.  If the use of airway

designators is common, I will add that feature to the program.

So far all the messages from Oakland have included a list of

fixes, and no airway designators.

Some PACOTS files contain more than one track with a given

designator.  The last one in the file is displayed.  It might

be better to display the one with the latest date.

2.2 PROCEDURE FILES

-------------------

Changes for PS1 version 1.3:

The procedure files in PS1 1.3 appear to have the same format

as 1.2.

The fix/navaid/airway file is encrypted, and can not be read

by NatPlot.

The airport file is not encrypted, and can be read by NatPlot.

If you have PS1 1.2, NatPlot can display the fix and navaid

information from that version (control file entry:  PS12DIR).

Selecting FILE | READ SID/STAR/APP... should get you a dialog box

showing the airport folders in your DEPARR folder.  If not, either

you have an incorrect 'Path' entry in NATPLOT.CTL, or you have

changed the normal location of the PS1 files.

Double-click on an airport folder to see the list of procedures

for that airport.  Double-click on a procedure to draw it.  The

next time you use this file-open dialog box (during the current

run of the program), you will start in the same folder.  Use the

"up one level" button in the dialog box to get back to the parent

folder to access other airports.

The FILE | ROUTE... option operates in a similar manner, except

the routes are all in one folder.

In a route file, the route line starts at the origin airport, but

not at a particular runway, since that information is not stored

in the file.  The route line is not drawn from the last fix to

the destination, since some people have included an approach and

missed approach in their route files.

The initial map center and range will be automatically selected

based on the min/max latitude and min/max longitude of the fixes

in the procedure (sort of).  You may then modify the center, range,

and projection as with other maps.

If DATA | WAYPOINT DATA is unchecked, the fix name and numeric type

will be shown next to the fix symbol.  If it is checked, the

lat/long and speed/alt restrictions will also be displayed.

To reduce clutter, detailed data for RWxx and MMxx waypoints are not

shown.  Note that if only RW or MM is used, data will be shown, since

those may be navaid names.

A solid blue line shows the flight path in procedure files.

A blue dashed (great circle) line connects the origin and

destination airports in a route file.

A light gray dotted line shows the course line from the reference

navaid for leg types 31, 32, 41, 42, 51, 52, 81, and 82 (heading

until crossing or intercepting).  For headings that do not intercept

the specified course, the line is drawn in both directions from the

reference navaid.  See section 4.5.

2.3 PRINTER

-----------

NatPlot was developed using a 600-dpi black and white printer,

a 300-dpi color printer, and a 1024x768 video display.

If you do not have a color printer, many of the line types will

look the same, and the facility boundaries show up a thick black

lines.  You may prefer the Black & White output scheme, which

uses several types of dotted/dashed lines.  Change this with

DISPLAY | PRINTER - USE COLOR.  A control file entry can be used

to set the start-up value so you do not have to change it on

each program run (Color Printer  0).

I want most maps printed in landscape orientation.  The program

starts in that mode.  The orientation can be changed either with

items on the FILE menu, or with the Printer Setup dialog box.

I may add an environment variable to control the start orientation.

Page margins are ignored and the full page size is always used.

2.4 DISPLAY

-----------

Changing an item on the Display and Data menus will cause a

repaint if AUTO REDRAW is checked.  I think.  I always leave

Auto Redraw unchecked now, and use the F5 key to do a repaint.

The display window will be repainted regardless of Auto Redraw

status whenever a Nat Track or procedure file is read, or items on

the Map or Projection menus are selected.

DISPLAY | CENTER and DISPLAY | RANGE may be used to change (of all

things) the center and range.  Control/C and Control/R take you

directly to the input dialogs.

Valid responses for the new map center are the same "recognizable

fixes" discussed in section 2.1, plus the airports in your PS1

airport files.  Other PS1 identifiers are NOT valid here.

Valid ranges are from 0.1 to 11000 nautical miles (0.2 to 20372

kilometers).  Invalid ranges are ignored with no error message.

Use DISPLAY | VIDEO - USE COLOR to control whether the color or

black & white scheme is used on the screen.  See also section 2.3.

The CLEAR, INFO, DISTANCE, and INTERSECTION functions are

explained in section 3.1.

The toolbar (top) and status bar (bottom) can be switched on/off.

2.5 MAP

-------

There are several pre-defined center/range/projection combinations.

Predefined maps also set the latitude at which longitude labels

are displayed, and the longitude at which latitude labels are

displayed.  If you change the map center, the labels remain where

they were.

There are two related menu items that may cause confusion.

MAP | GLOBE now sets a range of 3200 miles with an orthographic

projection.  (It used to change the map center too.)

PROJECTION | ORTHOGRAPHIC sets the projection, but not the center

or range.

The "Show Parameters" switch controls informational text at the

edges of the display.  The time printed will always be shown.

2.55 TEST PATTERN

-----------------

The test pattern contains samples of fixed pitch fonts available

on your computer, and color samples.

You may prefer fixed pitch fonts for the Message Line and possibly

for fix names and other map data.  Use the fixed pitch font names

listed in the test pattern for the control file items MapFont

and MessageLineFont.  (You may also use variable pitch fonts.)

This item still needs more work.

2.6 PROJECTIONS

---------------

Equations for the map projections are from "Map Projections -

A Working Manual, U.S. Geological Survey Professional Paper 1395",

by John P. Snyder, 1989.  Recommended:  an 'A' in high school

trigonometry.

This book was really cheap around 1989.  In 1999, USGS listed

it at $32, and Amazon had it at $42, but out of print.  Hint -

large public libraries.

Over two dozen projections are discussed, including forward

and reverse equations, sample computations, common uses,

and a bit of history about the creator.  383 pages.

The Lambert projection is used for many aeronautical charts.

For areas that are not "too big", straight lines are great

circles.  You can select a range of up to 11000 nautical miles

in this program, but expect distortion at the larger ranges.

See also section 4.2.

In the menu selection 'Lambert 30N/45N', the 30N and 45N refer

to "standard parallels".  The projection is based on a cone

which has the apex over a pole, and slices through the earth's

surface at the two standard parallels.

The program automatically changes the standard parallels when the

center latitude of the display changes.  You can then change them

again if you wish.  One reason for doing that may be to allow lining

up a bunch of pages on the wall of your computer room.  (Don't tell

your wife where you got the idea.)

The Orthographic projection looks like a globe, at least at larger

ranges.  A range of 3200 nautical miles (6000 kilometers) will fill

the display with one hemisphere.

2.7 DATA

--------

Various items on the display can be turned on or off.  Some

items are not displayed on some maps regardless of the settings

(e.g. - CEPAC routes are not shown on a SID).

747*.* files.  These are your PS1 (1.2) waypoint, station (Vor, etc.),

and (1.3) airport data files.

Fix Symbols.  When selected, a symbol will be shown at the waypoint

or station location, with the text moved down and right.  (There is

a separate switch on the Dafif menu for symbols at Dafif points.)

Route Distances.  When selected, computed distances between points

on fixed and flex tracks will be shown.  Fixed track airway names

are also displayed.  (to do - separate switches for airway

designators and flex track designators.)

Route Bearings.  When selected, computed magnetic bearings will be

shown (true bearings in polar regions).

********************************************************************

* Note - if the fixes are too close together (in terms of pixels,  *

* not miles), the bearings or distances will not be shown.  Select *

* a smaller range.                                                 *

********************************************************************

The numbers are rotated to parallel the route segment.  Rotated

screen fonts do not look as good as rotated printer fonts.

The values are computed based on the lat/long of the waypoints,

and are not taken from any official charts.  They are generally

within two degrees and two miles of the charted values.

NatPlot fixes.  Airports, fixes, and navaids needed for the

oceanic tracks are defined internally.  Obsolete fixes are from

airways that are no longer in use, but may be useful to you.

See file NATPLOT.FIX.

Boundaries for most oceanic ATC facilities are stored.

Coastline data is from WORLD10 available at Oak Software Repository,

and does not appear to have any copyright associated with it.

I have changed some of the original data, mostly in the Pacific.

If anybody knows of a small database of rivers and political

boundaries in a digital format, let me know.

A Lat/Long grid is available at 5 degree intervals.  Labels are

shown at fixed positions on pre-defined maps.  If the grid is

selected, it will be drawn on Sids/Stars also, where it may not

be obvious what the "extra" line is.

Tick marks can be shown at 1 deg Lat/5 deg long, or

1 deg Lat/1 deg Long intervals.

Magnetic Variation at 1/3 (display) inch intervals can be shown.

Waypoint Data controls the amount of data displayed at each fix

in procedure files.  Details in section 2.2.

KILOMETER DISTANCES can be used to switch between nautical miles

and kilometers.  The control file entry 'Distance KM' will set

kilometers at program start up.

2.72 USER.TXT

-------------

This file allows you to display your own lines and text in

various formats.  More to come.

For now, USER.TXT must be in the same folder as NATPLOT.EXE.

Command summary:

# comments begin with '#'

NoDraw

Draw

# Ignores commands after NoDraw until next Draw.

FontSize <points>

Color <red> <green> <blue>

# values 0 to 255.

# "Color 255 0 0" produces red.

Align  <left, center, right, top, bottom>

# "Align Left Top" works too

Line  <type> <lat/long> <lat/long>

# <types> are defined by Windows - solid, dash, dot,

# dashdot, dashdotdot

# <lat/longs> may also be 5 character FMS format.

Text     <lat/long> <text>

TextBox  <lat/long> <text>

# Text is printed horizontally at given position in

# current color and alignment (BOTTOM ignored for now).

TextR    <start lat/long> <end lat/long> <dist> <text>

TextBoxR <start lat/long> <end lat/long> <dist> <text>

# Rotated text (like an airway label), with or without box.

# Text is placed <dist> miles (centered) from the start

# position along a great circle route to the end position.

# The text is drawn parallel to the route at that point.

# If <dist> = 0, the label is drawn at the mid-point.

TextStack  <lat/long> <text>

Stack                 <text>

# TextStack defines start position, and first line of text.

# Multiple Stack command may follow, producing left justified

# text anchored to first line.

Arc  <lat/long> <radius> <true start bearing> <true end bearing>

# Draws an arc centered on the <lat/long>, with a radius

# of <radius>, clockwise from the start to end bearings.

# For over-the-top, use 2 entries (e.g. 340-360 and 000-040).

2.75 TERMINATOR

---------------

The terminator (sunrise-sunset line) and related information

can be displayed.

Separate switches are provided for:

terminator at designated time (yellow, wide)

civil twilight at designated time (yellow, narrow)

terminator at hourly intervals for next 6 hours (blue, wide)

The designated time may be the current time as provided by your

system (enter NOW), or a time you specify (enter date and UTC

hour). The format for entering a time is the hour followed by

the letter 'Z', a space, and the date in day/month/year format

(08Z 22/11/1999).  That's day/month, not month/day.

If a "next 6 hour" terminator line crosses the display boundary,

the time associated with that line will be shown in the format

RxxZ or SxxZ.  'R' = rise, 'S' = set, xx = UTC hour.

Bug - R/S labels may not show up on the printer.

Equations are from Astronomy with your Personal Computer, by

Peter Duffet-Smith, 1985.

The terminators are sea level positions.  I may add a 'FL350' switch.

2.76 TIME ZONE

--------------

NatPlot uses the Time Zone information from Windows to compute

UTC for displaying the terminator if the Data | Terminator |

Use Current Time option is selected.

You probably set your Time Zone when you first set up your

computer.  If not, use Start | Settings | Control Panel | Date/Time

and go to the Time Zone tab.  Select your time zone from the drop

down list.

This action sets an entry in the Windows Registry.  I did not find

any documentation on this subject -- I discovered it by using

the search function in regedit.exe.  (If you use regedit, be VERY

careful.  You may make your system un-bootable if you accidentally

change something.)

NatPlot looks for the registry entry HKEY_LOCAL_MACHINE,

System\CurrentControlSet\Control\TimeZoneInformation,

ActiveTimeBias. There is a chance that Microsoft changed the

details in versions after Windows 98.  (Now, where did I put that

book on Cobol ...)

2.77 Y2K

--------

During development of the terminator functions, I noticed a

Y2K bug.  The problem was in the TDate and TTime classes in

Borland's C++ Class Library.  They posted a patch that seems

to correct the problem.

This bug may also cause incorrect "time printed" info in

NatPlot versions 0.9 and earlier.

2.8 DAFIF

---------

The Digital Aeronautical Flight Information File is updated

every 28 days, and takes over an hour to download with a dial-up

connection.  However, only a few Mb of the 100+ Mb of data are

needed.

Initially, DAFIF (and other) data was written to binary files

to permit faster display speeds.  This isn't much of an issue

today, so the program is being changed to use text format files.

I plan to have NatPlot read the original DAFIF files for those

of you who have high-speed connections.  I will also continue

with the smaller files (in text format), for those with dial-up

connections for at least the near future.  It may turn out that

reading the original files is too slow for displaying airway

details, and a combination of original and processed files

will be needed.

Numerous on/off switches control the display of data from the DAFIF

database.  You can change multiple switches at once via the

Properties menu item.

The numbers after the airport selections refer to runway length.

ORTCA = Off Route Terrain Clearance Altitudes.  These give a 1000,

2000, or 3000 foot clearance depending on the region.

Code letters after other items are as coded in the database.  Some

meanings are high, low, both, RNAV, and terminal.  There will be

some reorganization of the data types.  "U" means "unknown", and

is not necessarily an NDB.

Most items display a large amount of data, and redrawing the display

can take several seconds on slower machines.  If you want to turn on

several items at once, use the DAFIF tab on the Properties page.

See also section 4.8.

There are several programming techniques that will improve display

speed, and that will be done after the conversion to text format is

complete.

The DAFIF data has some surprises.  Waypoints for RNAV approaches

are not included, but FMS Sid waypoints are.  A few segments of

high altitude airways in North America are of type "B" instead of

"H", and won't display unless both of those items are selected on.

Also, you will not find waypoint names created by private

navigation providers (e.g. Jeppesen).  For example, N7000/W06000

may be accessible with 060A in an FMS, but the official designator

is 75YCY.  Check that --- "was 75YCY".  Now it is 7060N.  Several

fixes in this part of Canada now are defined using 5 character FMS

format, but others are not.

The Special Use Airspace items are disabled in NatPlot, until I

get around to setting up the necessary text format files.  The

Sector Map program on my web site does show SUA.

2.8.1 RUNWAY DISPLAY

--------------------

Airports can be selected by longest runway length with the

four Airport xx-xx items.

If "Runways" is also checked, all runways for the selected

airports will be shown to scale.  If "Rwy Numbers" is selected,

the number will be printed next to the approach end of the runway,

but all are offset to the lower right.  I might be able to position

them "outboard" of other runways and rotated to be aligned with the

final approach course.

"Long Name" shows:

line 1 - identifier, field elevation, runway length / 100.

line 2 - first 30 characters of the airport name.

2.8.2 AIRWAY DATA

-----------------

Airways are categorized as low, high, or both.  The meanings vary

from country to country.  In the U.S., a few airways that seemingly

would be "high" are designated as "both".

Displaying airways will produce a noticeable pause.  Showing

all airway data might take 20 seconds with a moderate speed CPU.

Distances and bearings are those stored in the database (not

computed from lat/longs).  There are some obvious errors, some

of which persist between database cycles.  A bad value will

cause data to be drawn at an incorrect position because the

display position depends on the stored values.

Radials are displayed at a constant screen-distance from

navaids and waypoints.  The distance is a function of font

size. There is a trade-off here. If the radials are too close

to the navaid, they overlap each other, and are unreadable.

If they are too far away, they overlap the segment distances,

or may be on the wrong side of a nearby waypoint.

I picked a distance that works for "reasonable" map ranges.

Reasonable means a scale that is similar to what might be

found on a commercial chart.  That corresponds to a map

range of about 50 miles for a low chart, and 100 miles for

a high chart.

Note that a typical folded commercial chart has the same

area as about 8 printer pages.  Don't expect to use one piece

of paper to cover the same area as a commercial chart and

still have a readable map.

Airway names and distances between navaids or other end points

are shown below the airway, within a box, separated by a slash.

'J100/144' designates airway J100 with a 144 mile segment

distance.  A segment that is part of more than one airway would

be shown in this format:  J100-J128/144.

The boxed name/distance is shown once between navaids if

the airway is a straight line, and no other airways join or

split from the main airway.  If there are turn points or

other airways, additional boxed names/distances will be

shown between intermediate points.

At small ranges, some airway labels will be off-screen.  It

might be possible to fix that, at the expense of an even

slower display.

2.9 PROPERTIES

--------------

Properties is not a pull-down menu, but activates a property

sheet, or tabbed-dialog box, containing the on/off switches

that appear on other menus.

If you want to change more than one item at a time, it is easier

to do it here.  The display is redrawn when you select OK.

-------------------------------------------------------------------

|                    CHAPTER 3 - NON-MENU INFO                    |

-------------------------------------------------------------------

3.1 TOOL BAR

------------

Display control buttons:

decrease range

increase range

repaint display

Input window:

Enter 1, 2, or 4 fixes for use with computation buttons.

Valid fixes are only the "recognizable fixes" listed in

section 2.1 (no PS1 data).

Computation buttons:

Clear input window (eraser).

Look up fix coordinates (question mark).  A magenta line (great

circle) is drawn from the display center to the fix.

Compute distance (ruler).  Need 2 fixes, separated by a space.

Output is the true bearing, magnetic bearing, and distance (NM).

A magenta line (great circle) is drawn between the points.

(to do - KM distance if switch set.)

Compute intersection (intersecting lines).  Need 4 fixes.

Output is the lat/long of the intersection of a line from the

first fix to the second, with a line from the third fix to the

fourth.  Magenta lines drawn between the two pairs of fixes.

Distances use the ellipsoid model described in the URL section.

There are some inconsistencies with segment lengths that are

slightly less than 1/2 way around the Earth.

Output window:

You can type in this one too, but it's more useful to type in the

input window.

3.2 MOUSE TRACKING

------------------

The bottom border of the display (a.k.a. Message Line or Status Bar)

shows the mouse position as follows:

Horizontal : Vertical -- pixels from upper left

Latitude / Longitude

Magnetic Variation at mouse position

Great circle true course from map center to mouse position

Magnetic course from map center to mouse position using

variation at map center

Distance from map center to mouse position (NM or KM)

The default font for the message line is set by the run-time

library, which tries to find a variable-width sans-serif font.

With a variable-width font, the characters may jump left and

right as the values change.  See section 2.55 for help on

changing fonts.

3.3 HOT KEYS/CLICKS

-------------------

As with other Windows programs, underlined characters may be

used to activate menu selections  (Atl+M and O and 8 for

MAP | OCEANIC | 8 NORTH POLE).

F3 - decrease range.

F4 - increase range.

F5 - repaint the display.

F6 - clear input window.

F7 - look up fix in input window (1 item).

F8 - compute distance between fixes in input window (2 items).

F9 - compute line intersection (4 items).

Left click - adds lat/long to the input window for use with F8/F9.

Ctrl+C activates the Display Center dialog box (may need F5 to

redraw).

Ctrl+R activates the Display Range dialog box (may need F5 to

redraw).

Ctrl+S activates the Terminator Set Time dialog box (may need F5 to

redraw).

A right click in the map area will reset the display center to

that point.

3.4 MAGNETIC VARIATION

----------------------

Hardy posted a message in the fall of 1998 discussing PS1 file

formats and add-on programs.  He stated that the magnetic variation

file was off-limits.  (Hardy changed his position on this later,

but I'm not going to re-program that part of NatPlot now.)

NatPlot calculates variation on the fly, so to speak, using World

Magnetic Model-2000.  The web site documentation says the model

"consists of a degree and order 12 spherical-harmonic Main field

model comprised of 168 spherical-harmonic Gauss coefficients ..."

That works for me.

While this means the variation may agree with navigation data from

some sources, it will not necessarily agree with PS1.

Course computation equations use true courses.  Differences

between NatPlot and PS1 may appear anytime that a compass heading

is involved (e.g. Sid procedures, runway directions).

The PS1 nav display changes between true and magnetic modes at

73N and 65S.  NatPlot assumes headings in procedure files follow

the same plan.

3.5 PROGRAMMING

---------------

Most of the program is written in C++ (approx. 8000 lines), using

Borland's Object Windows Library (OWL), and the Standard Template

Library (STL).  The line drawing functions were originally written

in assembler, and converted to C in the 1980s.

-------------------------------------------------------------------

|                     CHAPTER 4 - PROBLEMS                        |

-------------------------------------------------------------------

4.1 SID CIRCLES

---------------

Related to the famous crop circles in England, I expect it

will eventually be proven that Sid Circles are alien spacecraft

routings to throw off ATC radar.  Until that proof is in, we can

also use the following explanations.

First, PS1 is more tolerant of what I regard as SID/STAR programming

errors than NatPlot is.  If a user has entered a Sid instruction

that says turn left direct somewhere, but the target fix is

actually to the right, NatPlot will draw a really big left turn.

Second, the turn radius will be different between the two programs.

In the case where there is a turn, a straight segment (e.g. until

a radial or until a DME), and a turn toward a fix, the end point of

the first turn, and therefore the straight segment, will be in

slightly different places.  The direction of the second turn may

thus be left in one program, and right in the other.

Third, the end points of a 'heading until altitude' segment will

be different, because the fixed climb rate I use is probably quite

different from what PS1 uses.

Fourth, there is a difference in magnetic variation computations,

which means that the ends of heading related segments will be in

slightly different places.  See also section 3.4.

Since sometimes a procedure actually contains a 270 degree turn

(or more), I picked a "wrong way" tolerance of 6 degrees.  If

NatPlot computes a heading change of less than 6 degrees, the turn

will be the short way.  More than 6 degrees will be in the direction

specified in the procedure.

4.2 EXTRA AND MISSING LINES

---------------------------

The problem with extra lines in Lambert projections has been

partially corrected.  They still appear at very large ranges.

Lines that have at least one end point well beyond the borders

of the display are still not drawn.  However "well beyond" is now

much larger than it was before.

In polar regions using the Lambert projection, data will be not be

displayed within a wedge at the top center of the display.  You can

see where this area is by turning on the grid.  You can avoid the

problem by using the orthographic projection in polar regions.

4.3 NAT/PACOTS TRACKS NOT DRAWN

--------------------------------

Make sure Nat Eastbound Routes and/or Nat Westbound Routes are

checked in the Data menu.  I may add a warning message later.

Use a text editor to look at the file you are trying to display,

and make sure lines are in the proper format (section 2.1 and 2.11).

There are now switches on the Data|Routes menu to control the

display of all routes, including PS1 procedure files.  The appropriate

switch is turned on when the corresponding item is selected from the

File menu.  It will remain on until you turn it off.

4.4 WAYPOINT DATA OVERPRINTS

----------------------------

If a procedure uses the same point more than once, data for each

pass over that point will print in the same place.  It is relatively

easy to keep track of where waypoint data blocks are printed, and

make sure later ones are repositioned if necessary.  I may do that

sometime.

4.5 COURSE INTERCEPTS THAT DON'T

--------------------------------

There are many user-contributed files that contain instructions of

the form "turn left heading 062 to intercept the 242 course ...".

The problem is, it is really difficult to intercept a course that

you are paralleling.

When NatPlot sees this, or anytime the heading does not intercept

the course within a reasonable distance, it just ignores the

intercept part, and stops at the end of the first turn.  It then

tries to processes the next instruction, which, if you are lucky,

will be a "direct to".

A related problem can occur with a "heading to intercept" leg

followed by a "heading to DME" leg.  If the intercept angle in the

first instruction is too shallow, you will have to backtrack to

the DME fix in the second instruction.

Another problem along this line is a missed approach "climb to

altitude" leg followed by a "heading to intercept" leg using a

navaid that is very close to the turn point.  The climb angle in

NatPlot is sort of a wild guess, so you may end up on the wrong

side of the navaid, causing the heading to diverge from the course.

PS1 seems to handle these situations without a whimper, and usually

draws "nice" route lines too.  However, the FMC leg directions and

distances may not correspond with the route.

Clarification.  In a flat coordinate system it is impossible to

intercept a course you are paralleling.  In a curved one, it is

just difficult.  Assuming your course is a great circle and your

heading is more or less a rhumb line, you may or may not intercept

depending on the initial conditions.  If you do intercept, you will

probably (a) run out of fuel, or (b) be beyond signal reception

distance.

4.6 ERROR MESSAGES

------------------

AIRPORT NOT IN DATABASE:  xxx

Sid/Star/Approach/Route file specifies an airport not in your

PS1 database.

BAD DD/MM/(YY)YY FORMAT

The date for the terminator display was not recognized.

BAD HEADING IN FILE AT WPT:  xxx

BAD IAS IN FILE AT WPT:  xxx

BAD LAT/LNG IN FILE AT WPT:  xxx

Procedure file not in expected format, or the file is not

a procedure file.  In some cases, the bad value will be

replaced with another value and processing will continue.

BAD HOUR FORMAT

The start time for the terminator display was not recognized.

Program expects one or two digits followed by an upper or

lower case 'Z'.

BAD SID/STAR/APP/ROUTE FILE FORMAT - WRONG RECORD LENGTH

A procedure file does not have the expected format.

CAN'T FIND 747WPT.DAT FILE:  xxx

The file was not found in the default or user-specified path.

CAN'T FIND AIRPORT INFO

Sid file specifies an airport not in your PS1 database.

CAN'T FIND END AIRPORT

Route file specifies an airport not in your PS1 database.

CAN'T FIND PATH xxx AIRPORT xxx FILE xxx

Couldn't find the file necessary to retrieve runway coordinates.

CAN'T FIND RUNWAY INFO

Sid file specifies a runway not in your PS1 database.

CAN'T FIND START AIRPORT

Route file specifies an airport not in your PS1 database.

CAN'T OPEN CONTROL FILE:  xxx

NATPLOT.EXE and NATPLOT.CTL must be in the same folder.

If you are using a shortcut to run the program, right click on the

NATPLOT icon, select 'Properties', then the 'Shortcut' tab, and

make sure the 'Start in' entry (e.g. C:\NATPLOT\) matches the path

part of the 'Target' entry (e.g. C:\NATPLOT\NATPLOT.EXE).

CAN'T OPEN FILE xxx

The file requested in the file-open dialog box could not be opened.

CAN'T OPEN REQUESTED FILE:  USER.TXT

Show USER.TXT was requested on the Data menu, but the program can't

find the file.  This message is only given once on each run of

the program.

NATPLOT.EXE and USER.TXT must be in the same folder.

FIX NOT FOUND xxx

An unknown fix was specified for the map center.  See section 2.4

for valid responses.

GET RWY COORDS:  RECORD TOO SHORT

Airport file not in the expected format.

GET RWY COORDS:  UNEXPECTED END OF FILE

Airport file not in the expected format.

LOOKUP() BEFORE READ()

Programming error.  You shouldn't see this bug, because I fixed

it (at least in version 0.7).  Oooops - You may have seen it again

in version 0.9 - same message - different cause.

NON-NUMERIC COURSE IN PROCEDURE (SET TO 0):  xxx

In a leg of type 31, 32, 41, 42, 51, 52, 81, or 82, the "crs"

item should contain a number, but does not.  NatPlot uses a course

of zero, and the path will intercept the 180 or 360 radial.  PS1

appears to ignore this error, and may just do a "direct to fix".

SKIPPING TXT/DOC FILE xxx

You have selected a file that is probably a text file where a

procedure file is required -- name has a TXT or DOC extension,

or is named README or READ.ME.

TOO MANY ERRORS - GIVING UP

After too many errors in a procedure file (currently = 3),

processing will stop.  The file may not be a procedure file.

WARNING:  FILE OPEN DIALOG ERROR - CAN'T ACCESS FILE

The file-open dialog was closed without using OK, CANCEL, or

ABORT.  This shouldn't happen.

YEAR OUT OF RANGE (1901-2036)

The year for the terminator display was not in the allowable

range.

4.7 FAILS ON START UP

---------------------

There are several known problems.  I'm sure there are others as well.

Users report that NatPlot will not run on NT/2000.  If that changes,

it is purely by accident.  One user has had success with Sector Map

on NT/2000, using the Sector Map version that now works on XP.  That

problem was caused by some change related to menu processing that is

internal to Windows.  The two programs share a lot of code, but I

don't know which parts are causing the problem.

Other users have had problems on XP if the path names in NATPLOT.CTL

refer to non-existent folders.  I should be able to catch that type

of error with some programming effort.  Until then, you can verify

the path names.

The program assumes that a printer has been set up.  I get caught

by this one when I get a new computer (not that often).  I should

be able to catch this one too.

4.8 SLOW DISPLAY UPDATES

------------------------

Font size matters.  On my computer, displaying all waypoints in

the DAFIF database on a globe view takes 10 seconds at font size 6,

and 2 to 3 seconds at font size 7.  There is not much speed

difference with the default North Atlantic map.

The default size is 7.  Adjust with the MapFontVideoSize item

in NATPLOT.CTL, then re-start the program.

-------------------------------------------------------------------

|                   CHAPTER 5 - COMING ATTRACTIONS                |

-------------------------------------------------------------------

This is a wish list, not in any particular order.

Add navaid details - freq, class, elevation, long name, declination

Use DAFIF for fix look-ups.

Add Special Use Airspace display.

Show track designator on NAT/PACOT tracks.

Show lat/longs of fixes on NAT/PACOT tracks.

Use data path from control file to find USER.TXT.

Text should be case-sensitive in all USER.TXT commands.

Add start-up page orientation to control file.

Add more pre-defined globe views.

Show KM distances in output window if switch set.

User defined map center/radius combinations accessible via menus.

Add control file entries for items on Data menu.

User defined colors for various map elements.  Some colors that

look great on the screen do not look good as single pixel-wide

lines on my color printer.  Your printer is probably different

than mine.

User positioning of lat/long labels.

Use page margins from the print setup dialog box.

Print 2, 3, or 4 charts per page.

Top of page direction - true north, magnetic north, course line,

runway heading, user defined, etc.

Set control file items from within program, with save-to-disk

option.

Move to Linux.

-------------------------------------------------------------------

|                CHAPTER 6 - RECENT VERSION HISTORY                |

-------------------------------------------------------------------

23    Added error message if user.txt requested, but not found.

22    Lat/Long tick marks at 1 deg/5 deg intervals (Data|Lines menu)

21    Fixed bug in ActiveTimeBias call for XP.

Added "synonym" category in NatPlot database to handle

items with a new name at the same location (e.g. FMN->RSK).

20    Changed airway file format.  Show DAFIF airway details - IDs,

radials, distances.

Fixed runway bug from version 18.

19    Show PACOTS tracks 14 and 15.  Update links.

18    Added group on/off buttons for Dafif data.

Added DAFIF Runway display.  There is a bug which prevents

the display of some runways at ranges greater than about

15 miles.

Changed Dafif waypoint file format.

17    Minor changes for debugging.

16    Minor changes to HTML processing.

Added a few fixes that appear in PACOTS routes.

15    Changed to simple version numbers instead of major.minor

Added DAFIF data display.  No look-ups yet.

Added USER.TXT file.

Yet another repair of the time/date functions.